FACULTY OF ENGINEERING,  COMPUTING AND THE ENVIRONMENT

School of Computing

MSc DEGREE

IN

Data Science

Name: Dipayan Chowdhury

ID Number: K2448522

Project Title: The Classification of Potholes from Motion Sensor Data

Date: 18/09/2025

Supervisor: Dr Farzana Rahman

WARRANTY STATEMENT

This is a student project.  Therefore, neither the student nor  Kingston University makes any warranty, express or implied, as to the accuracy of the data or conclusion of the work performed in the project and will not be held responsible for any consequences arising out of any inaccuracies or omissions  therein.

Table of Contents

1. ABSTRACT	3

2. INTRODUCTION AND BACKGROUND	3

3. Literature Review	5

3.1 Early Applications in Road Anomaly Detection	5

3.2 Broader Applications of IMUs in Anomaly Detection	6

3.3 Machine Learning for Pothole Detection	6

3.4 Sensor Positioning and Deployment Challenges	7

3.5 Crowdsourcing and Scalability	8

3.6 Ethical, Legal, and Practical Considerations	8

3.7 Contribution and Research Gap	8

3.8 Literature Review Summary	9

4. Methodology	10

4.1 Dataset Acquisition	10

4.2 Data Labelling and Annotation	10

4.3 Preprocessing	10

4.4 Feature Extraction	10

4.5 Model Development	10

4.6 Evaluation	11

4.7 Visualisation	11

5. Project Artefact	12

6. Implementation	13

6.1 Data Acquisition and Conversion	13

6.2 Preprocessing	13

6.3 Feature Extraction	13

6.4 Model Training	14

6.5 Evaluation	14

6.6 Deployment	14

7. Results and Evaluation	15

7.1 Pothole Dataset Results	15

7.2 UCI HAR Dataset Results	16

7.3 Comparison of Datasets	16

7.4 Evaluation of Model Selection	17

7.5 Limitations and Risks	17

7.6 Summary of Findings	18

8. References…………………………………………………..………………………………………………………….19

## 1. Abstract

Road surface degradation, particularly the formation of potholes, remains a significant challenge for transport authorities worldwide. Potholes reduce ride comfort, cause vehicle damage, and create safety hazards for drivers and pedestrians. Traditional pothole detection and monitoring strategies rely on manual inspection or specialised scanning vehicles, which are costly, labour-intensive, and infrequently deployed. Consequently, many defects remain unrecorded or unaddressed, contributing to rising maintenance costs and public dissatisfaction.

This project proposes the development of a supervised machine learning model to detect and classify potholes from smartphone motion sensor data. Accelerometer and gyroscope signals, augmented with GPS, will be used to identify vertical road anomalies. A bespoke labelled dataset will be constructed either through publicly available data repositories or through controlled data collection, with each instance annotated to distinguish potholes from similar road features such as speed bumps and manhole covers.

Preprocessing will include noise filtering, signal segmentation, and feature extraction in both time and frequency domains. Classical machine learning models such as Decision Trees, Random Forests, and Support Vector Machines will be implemented, with model evaluation based on accuracy, precision, recall, F1-score, and confusion matrices. The performance of these classifiers will then be compared to identify the most effective approach for pothole detection under varying conditions.

The anticipated outcome is a reliable classification system achieving high accuracy (>85%) while remaining computationally efficient and interpretable. Such a system would provide a scalable, cost-effective, and accessible method for councils and government agencies to monitor road conditions, prioritise maintenance, and enhance road safety.

## 2. Introduction and Background

Potholes are a pervasive and persistent challenge in modern road infrastructure. They form when water infiltrates cracks in the road surface, weakens the underlying structure, and is compounded by freeze–thaw cycles and heavy traffic loads. The result is a surface depression that not only compromises ride comfort but also presents significant safety hazards. The United Kingdom, like many developed countries, faces an escalating crisis in road maintenance. The Asphalt Industry Alliance’s Annual Local Authority Road Maintenance Survey (ALARM Report, 2023) estimated that local authorities collectively face a £14 billion backlog in road repairs, with more than one million potholes reported annually. In parallel, the RAC Foundation recorded a 30% rise in pothole-related breakdowns between 2017 and 2022, underscoring the urgency of the issue.

The economic costs are considerable. Potholes are estimated to cause hundreds of millions of pounds in vehicle repair costs each year in the UK alone, with suspension damage, tyre blowouts, and wheel misalignments being common consequences. Insurance companies are increasingly burdened by claims, while councils face mounting criticism for failing to address defects promptly. These costs extend beyond direct repairs: poor road quality reduces the efficiency of freight and logistics, increases fuel consumption, and diminishes public confidence in transport infrastructure.

The range of stakeholders affected is extensive. Local councils and highway authorities are primarily responsible for road upkeep, yet they often struggle to balance limited budgets against competing priorities. Citizens are impacted both directly, through vehicle damage and accidents, and indirectly, through delays and reduced road safety. Insurers face higher claim volumes and payouts, while national governments must contend with the broader economic impacts of neglected infrastructure. Even businesses, particularly those reliant on logistics, suffer reduced efficiency due to poor road conditions.

Traditional pothole monitoring methods, although reliable, are insufficient to meet the scale of the challenge. Manual inspections by maintenance crews are labour-intensive, inconsistent, and reactive, typically initiated only after public complaints. Specialised survey vehicles equipped with laser scanners or high-resolution cameras provide accurate data, but they are prohibitively expensive to operate on a regular basis and cover limited geographic areas at infrequent intervals. These methods, while important, create a lag between defect formation and repair, leaving roads unsafe and stakeholders dissatisfied.

In recent years, advances in mobile technology have opened new possibilities. Smartphones are ubiquitous, relatively inexpensive, and equipped with powerful sensors including accelerometers, gyroscopes, and GPS receivers. These sensors are capable of recording tri-axial motion data at high frequencies, capturing even subtle vibrations and jolts. Several research studies have shown that accelerometer data can detect the vertical shocks associated with potholes, while gyroscopes add complementary information on rotational movements, enhancing classification accuracy. By leveraging these sensors, it is possible to crowdsource road condition monitoring, creating a distributed sensing network that is both scalable and cost-effective.

However, challenges remain in making such systems reliable and practical. Motion sensor signals are inherently noisy, influenced by factors such as road texture, driving style, vehicle suspension, and sensor placement within the car. Furthermore, other road features such as speed bumps, manhole covers, and railway crossings can produce accelerometer patterns that are similar to potholes, complicating classification. Threshold-based methods, which simply flag anomalies exceeding a predefined value, often fail to discriminate effectively between these features. Supervised machine learning, by contrast, offers a way to learn the distinguishing patterns between road anomalies, but requires carefully labelled training data and robust feature extraction methods.

The research problem addressed in this project can therefore be stated as follows: How can smartphone motion sensor data, when processed through supervised machine learning, be effectively utilised to distinguish potholes from other road anomalies in a manner that is accurate, scalable, and cost-effective for councils and road authorities?

The significance of this research is manifold. It aligns with broader trends in smart cities and intelligent transport systems, where data-driven approaches are increasingly used to enhance urban mobility and infrastructure management. By developing a proof-of-concept pothole detection system, this project demonstrates how low-cost consumer technology can be repurposed for civic benefit. The potential benefits include improved road safety, reduced repair costs, more efficient allocation of maintenance resources, and increased transparency for stakeholders.

This project is situated at the intersection of applied artificial intelligence, sensor technology, and infrastructure management. It builds on prior work in anomaly detection using IMUs, adapts lessons from related fields such as healthcare and wearable technology, and applies them to the pressing societal challenge of road maintenance. In doing so, it seeks to contribute not only technical insights but also practical outcomes that may inform policy and practice in the management of road networks.

# 3. Literature Review

The detection of road surface anomalies using low-cost, widely available sensors has emerged as an active area of research, particularly in the context of smart cities and intelligent transportation systems. Traditional approaches to road monitoring, such as manual inspections and dedicated survey vehicles, remain limited by cost, scalability, and timeliness. In contrast, the increasing ubiquity of smartphones has provided a new opportunity to harness embedded inertial measurement units (IMUs), including accelerometers and gyroscopes, for distributed and crowdsourced anomaly detection. This literature review critically examines the evolution of pothole detection systems, evaluates the role of machine learning in improving classification accuracy, and connects insights from related fields such as healthcare and wearable technology.

## 3.1 Early Applications in Road Anomaly Detection

Initial attempts to employ smartphones for pothole detection focused on simple thresholding methods. Mednis et al. (2011) were among the first to demonstrate that accelerometer data from Android devices could identify vertical shocks caused by road irregularities. Their approach relied on detecting sudden peaks in acceleration values that exceeded predefined thresholds. While pioneering, this method suffered from significant limitations: it was sensitive to vehicle type, suspension stiffness, and driving style, and often generated false positives from events unrelated to potholes, such as sudden braking or cornering.

Building on this work, Araujo et al. (2019) integrated accelerometer signals with GPS data to both detect and geolocate anomalies. Their system applied decision tree classifiers, achieving moderate improvements in accuracy compared to thresholding. However, the study emphasised the variability introduced by different vehicles, as identical potholes produced different acceleration magnitudes depending on suspension characteristics. This highlighted the need for approaches capable of generalising across heterogeneous conditions.

Zhang et al. (2017) took a significant step forward by incorporating gyroscope data alongside accelerometer signals. Using Support Vector Machines (SVMs), they achieved classification accuracy of 86%. Gyroscopes added discriminative power by capturing rotational movements, which helped distinguish between lateral and vertical disturbances. Nonetheless, their study was limited by its reliance on manually engineered features and evaluation within a single vehicle, raising concerns about robustness in real-world deployments.

Gupta et al. (2020) shifted focus towards scalability by crowdsourcing data from multiple drivers. Their system aggregated accelerometer and GPS signals across different devices and vehicles, demonstrating the feasibility of large-scale monitoring. The aggregation process reduced noise caused by individual vehicle variability. However, their approach was primarily designed for detecting anomalies at the network level rather than delivering accurate per-vehicle classifications.

Critique: Early systems established proof-of-concept, showing that consumer-grade sensors could identify pothole-like anomalies. However, thresholding methods lacked flexibility, while early machine learning systems were constrained by limited datasets and narrow evaluation contexts. These limitations underscore the importance of robust feature extraction, diverse data collection, and adaptable models.

## 3.2 Broader Applications of IMUs in Anomaly Detection

IMUs have found wide application in domains outside transportation, particularly in healthcare and human activity monitoring. These studies provide valuable insights into anomaly detection from time-series signals.

Bourke et al. (2010) demonstrated that combining accelerometers with gyroscopes improved the accuracy of fall detection in older adults compared to using accelerometers alone. Their threshold-based system achieved high sensitivity but highlighted the importance of sensor placement on the body. Similarly, Salarian et al. (2004) used IMUs to assess gait in Parkinson’s disease patients, showing that movement irregularities could be reliably quantified with wearable sensors.

In cardiovascular monitoring, Lahdenoja et al. (2017) employed smartphone accelerometers and gyroscopes to detect atrial fibrillation, achieving 97.4% accuracy. This work illustrated the potential of consumer devices in capturing subtle physiological anomalies through signal processing and classification techniques.

Batool et al. (2020) extended IMU applications into smart homes, using accelerometers and gyroscopes to recognise daily activities such as walking, sitting, and lying down. Their feature optimisation approach emphasised that both time-domain and frequency-domain characteristics are necessary for accurate classification.

Comparison: Across healthcare and wearable studies, IMUs have consistently been shown to capture meaningful anomaly signatures when supported by robust preprocessing and classification algorithms. These findings parallel the requirements for pothole detection, where sensor fusion, feature diversity, and preprocessing are critical for overcoming noise and variability.

Connection: Lessons from these domains suggest that adopting both time-domain and frequency-domain feature extraction, combined with sensor fusion, will significantly enhance classification accuracy in road anomaly detection.

## 3.3 Machine Learning for Pothole Detection

The limitations of thresholding approaches prompted researchers to adopt supervised machine learning techniques.

Hussain et al. (2019) used smartphone accelerometers to assess road surface roughness, applying Random Forest classifiers. Their model achieved over 85% accuracy, demonstrating that feature-based machine learning was both effective and practical in small-to-medium-sized datasets. Similarly, Mistry et al. (2019) applied Random Forest classifiers to classify cycling activities using smartphone sensors, underscoring the adaptability of these models to other motion-related domains.

Islam et al. (2021) introduced a hybrid deep learning approach combining convolutional neural networks (CNNs) with long short-term memory (LSTM) networks. This model achieved over 90% accuracy in classifying potholes. The CNN component captured spatial patterns in IMU data, while the LSTM component modelled temporal dependencies, which are critical in sequential sensor data. However, the approach required a large labelled dataset, which may not be feasible in smaller projects or early deployment stages.

Matarazzo et al. (2021) similarly applied CNNs to raw accelerometer data, achieving accuracy above 90%. By bypassing manual feature engineering, CNNs simplified the modelling pipeline. Nonetheless, deep learning models presented challenges of interpretability and computational intensity, raising concerns about deployment in resource-constrained environments.

Contrast: Classical models such as Random Forests and SVMs remain competitive where datasets are limited and interpretability is valued. Deep learning models offer higher accuracy with large datasets but sacrifice transparency and require substantial computational resources. For projects constrained by data availability, classical methods may be more appropriate.

## 3.4 Sensor Positioning and Deployment Challenges

The placement of smartphones within vehicles is a significant source of variability in detection systems.

Mistry et al. (2019) observed that sensor orientation influenced classification outcomes in cycling activity recognition. In the vehicular domain, smartphones placed on dashboards, cup holders, or seats capture different signal profiles. Swayze and Johnson (2015) demonstrated in balance monitoring that accelerometer and gyroscope accuracy is highly dependent on consistent sensor orientation.

Hernandez et al. (2014), in their work on physiological monitoring with Google Glass, highlighted that consumer devices vary in sensor quality and calibration, further complicating generalisation. For pothole detection, differences in smartphone brands, operating systems, and mounting positions present similar challenges.

Critique: Many studies inadequately report sensor placement, reducing reproducibility. Addressing this limitation requires systematic documentation and possibly model calibration for different placements.

Connection: For practical deployment, future systems must either standardise sensor positioning (e.g., dashboard mounts) or incorporate orientation-independent features that mitigate placement variability.

## 3.5 Crowdsourcing and Scalability

Large-scale monitoring requires methods that extend beyond individual vehicles.

Gupta et al. (2020) explored a crowdsourced approach, aggregating smartphone data from multiple users to generate city-wide pothole maps. Their results demonstrated that collective sensing improved robustness, as noise from individual devices was averaged out.

However, crowdsourced approaches raise questions about data quality, user compliance, and privacy. Aggregated datasets may include inconsistent labelling, while varying driver behaviour and vehicle types can introduce additional noise. Ethical concerns also emerge, as GPS-based tracking of vehicles must comply with data protection regulations such as GDPR.

Critique: While promising for scalability, crowdsourcing strategies require rigorous quality control and privacy safeguards to ensure both reliability and ethical compliance.

## 3.6 Ethical, Legal, and Practical Considerations

The deployment of smartphone-based pothole detection systems intersects with ethical and legal issues.

GDPR compliance necessitates anonymisation of GPS data and informed consent from participants in data collection. Islam et al. (2021) addressed this by anonymising location traces in their dataset. Safety is another concern: drivers must not be distracted during data collection. Most studies mitigate this by assigning annotation duties to passengers or using automated labelling from external data sources.

Practical deployment also requires balancing accuracy with interpretability. Councils and road authorities are more likely to adopt models that are transparent and explainable, even if marginally less accurate, to justify maintenance decisions to stakeholders.

Connection: This project acknowledges these concerns by prioritising interpretable models and adopting strict ethical protocols during data collection, ensuring compliance with legal standards while maintaining practical applicability.

## 3.7 Contribution and Research Gap

This review highlights several critical gaps in the current literature:

- Vehicle-specific variability: Few studies explicitly address differences in suspension systems, driving styles, and vehicle dynamics. This reduces the generalisability of detection systems.

- Dataset limitations: Many studies use datasets collected under narrow conditions, limiting their applicability across broader contexts.

- Deployment feasibility: Issues of sensor placement, device variability, and data privacy remain underexplored, hindering real-world implementation.

This project aims to address these gaps by developing a labelled dataset tailored to multiple anomaly types, employing supervised machine learning models suitable for smaller datasets, and critically evaluating results in terms of both accuracy and interpretability. Unlike studies that focus solely on maximising accuracy, this research emphasises practical deployment, transparency, and scalability for councils and road authorities.

## 3.8 Literature Review Summary

The body of literature reviewed demonstrates that smartphone-based IMUs are a viable foundation for pothole detection. Early threshold-based approaches established feasibility but lacked robustness. Classical machine learning models have since emerged as effective tools for classification in limited data contexts, while deep learning models promise higher accuracy but require significant resources.

Across domains, the consistent lesson is that sensor fusion, robust preprocessing, and a balance between accuracy and interpretability are key to success. Ethical considerations, particularly regarding data privacy and user safety, are increasingly recognised but require more systematic attention.

The contribution of this project lies in bridging the gap between theoretical advances and practical deployment. By creating a labelled dataset, applying interpretable supervised models, and visualising results in a form usable by councils, this project directly connects academic research to societal needs.

## 4. Methodology

The methodology of this project follows a structured pipeline consistent with best practice in supervised machine learning applications. It encompasses dataset acquisition, preprocessing, feature extraction, model development, evaluation, and visualisation.

### 4.1 Dataset Acquisition

The primary dataset for this study will be obtained from either publicly available repositories, such as the IEEE DataPort “Smartphone Sensor Data for Road Anomaly Detection”, or relevant Kaggle datasets containing tri-axial accelerometer, gyroscope, and GPS signals. Should these datasets prove insufficient in scope or quality, supplementary data collection will be undertaken. In this case, a smartphone will be securely mounted within a vehicle, with one person driving and another acting as annotator. The annotator will record the occurrence of anomalies such as potholes, speed bumps, and manhole covers in real time, providing ground-truth labels for subsequent analysis.

### 4.2 Data Labelling and Annotation

Accurate labelling is critical to supervised learning. During self-collected data gathering, anomalies will be categorised into predefined classes. The time-stamped annotations will then be aligned with the corresponding IMU signals to create a labelled dataset suitable for model training. The potential subjectivity of manual labelling will be mitigated by adopting clear annotation protocols and, where possible, cross-validating labels with repeated passes over the same road segments.

### 4.3 Preprocessing

Raw IMU data is inherently noisy, containing artefacts from road texture, engine vibrations, and environmental interference. Preprocessing will involve applying digital filters such as low-pass and moving average filters to reduce noise, normalising the signals to account for scale differences, and segmenting the continuous data into overlapping windows of one to two seconds. Each window will be assigned a label corresponding to the recorded anomaly type.

### 4.4 Feature Extraction

Building on prior studies (e.g., Zhang et al., 2017; Hussain et al., 2019), both time-domain and frequency-domain features will be extracted. Time-domain features will include measures such as root mean square (RMS) acceleration, mean, standard deviation, maximum, minimum, and signal magnitude area. Frequency-domain features will be derived using fast Fourier transform (FFT), enabling analysis of dominant frequencies and spectral entropy. By fusing accelerometer and gyroscope features, the feature set will provide richer discriminatory power than accelerometer data alone.

### 4.5 Model Development

Three supervised classifiers will form the core of the experimental pipeline: Decision Trees, Random Forests, and Support Vector Machines. These algorithms were chosen for their balance of accuracy, interpretability, and computational efficiency. Hyperparameter optimisation will be performed using grid search and cross-validation. While deep learning approaches such as convolutional neural networks (CNNs) and long short-term memory networks (LSTMs) have demonstrated strong performance in related studies, they require larger datasets and are less interpretable. Nevertheless, a lightweight CNN may be implemented as a comparative benchmark.

Alternative models, including K-Nearest Neighbours (KNN), Naïve Bayes, and logistic regression, will also be briefly explored as baselines. These models provide useful points of comparison, though they are not expected to outperform ensemble methods or SVMs in handling the complexity of IMU signals.

### 4.6 Evaluation

Model performance will be assessed using multiple metrics: accuracy, precision, recall, and F1-score. Confusion matrices will be used to visualise misclassification patterns, particularly the extent to which potholes are confused with speed bumps or other anomalies. Receiver Operating Characteristic (ROC) curves will be generated to evaluate trade-offs between sensitivity and specificity. Evaluation will follow a five-fold cross-validation strategy, ensuring that models generalise beyond specific subsets of data.

### 4.7 Visualisation

To demonstrate practical applicability, predicted pothole locations will be mapped using GPS data. Python libraries such as Folium or GeoPandas will be used to create interactive maps highlighting the spatial distribution of detected anomalies. This will provide a visual representation of how councils could deploy the system to identify high-priority road segments.

4.8 Contingencies

If dataset size proves inadequate, data augmentation techniques will be considered. For example, adding synthetic noise or jittering signals can simulate variations in driving conditions. If models underperform, feature engineering will be revisited, and hybrid approaches combining classical and deep learning models may be explored. Ethical and legal considerations will be addressed throughout, including informed consent for participants in data collection and anonymisation of GPS data to comply with GDPR.

By combining rigorous data collection, feature-rich modelling, and careful evaluation, the methodology ensures that the project produces robust, reproducible, and practically relevant results within the constraints of an MSc research timeline.

# 5. Project Artefact

The artefact developed for this project is a machine learning pipeline implemented in Python, designed to detect and classify potholes from smartphone motion sensor data. This artefact constitutes the core practical outcome of the research, demonstrating the feasibility of automated pothole detection using widely available sensing technologies such as accelerometers and gyroscopes. The design of the pipeline ensures not only that it meets the objectives of the current project but also that it can be readily adapted for use with other datasets in the future, thereby enhancing its long-term utility. At its foundation, the artefact addresses the key research problem: differentiating between true potholes and other road surface anomalies that may produce similar vertical motion responses, such as speed bumps or manhole covers. To this end, the pipeline follows a structured workflow, comprising data acquisition, preprocessing, feature extraction, model training, evaluation, and deployment.

The dataset used for experimentation originates from the publicly available Smartphone Sensor-Based Road Surface Monitoring dataset, which contains accelerometer and gyroscope readings collected from real driving scenarios. The dataset provides labelled ground truth for different road conditions, enabling supervised training of classification models. To accommodate variations in data storage formats, the artefact begins with a conversion step in which MATLAB .mat files are processed into structured .csv files. These are then merged into a unified dataset for analysis.

A central design decision was to keep the pipeline modular and generic. Each stage - conversion, preprocessing, feature extraction, classification - is encapsulated in such a way that if a new dataset with a compatible structure (accelerometer and gyroscope time series with labels) is provided, it can be integrated with minimal modification. For example, the conversion module is flexible enough to handle varying key names in .mat files, while the feature extraction function allows the user to specify window sizes and overlaps, making it possible to adapt to different sampling frequencies and driving conditions.

The classification stage incorporates three widely used supervised learning models: Decision Trees, Random Forests, and Support Vector Machines. These were selected not only because they are established techniques in time-series classification but also because they provide complementary strengths. Decision Trees are interpretable and computationally efficient, Random Forests offer robustness and reduced overfitting through ensemble learning, and Support Vector Machines are effective for handling non-linear class boundaries.

The artefact includes facilities for both traditional train-test splitting and k-fold cross-validation, allowing a more rigorous evaluation of generalisation performance. Furthermore, it integrates visualisation components that plot raw sensor signals, confusion matrices, and GPS-based maps of detected potholes. These visualisations enhance interpretability for stakeholders such as local councils, who may wish to inspect not only the classifier’s outputs but also their geographical distribution.

Finally, the artefact supports model persistence, saving trained classifiers in a portable .pkl format. This ensures that a trained model can be deployed on other platforms, such as a mobile application or an embedded system within a vehicle.

In summary, the project artefact is a complete, modular, and extensible pipeline for pothole detection from motion sensor data. It demonstrates the feasibility of the approach, provides tangible outputs that can be evaluated against the stated aims, and establishes a foundation for future extensions, such as real-time processing or integration into navigation systems.

6. Implementation

The implementation of the artefact followed a phased approach, consistent with the methodological framework outlined earlier in this dissertation. This section details the practical steps taken in developing the pipeline, including justifications for key design choices, descriptions of the software and resources employed, and an account of the challenges encountered.

## 6.1 Data Acquisition and Conversion

The first challenge addressed was the conversion of raw sensor data. The dataset chosen for this study provided data in MATLAB .mat format, structured as multi-dimensional arrays containing accelerometer and gyroscope readings. While this format is compact, it is not immediately compatible with Python-based data science workflows.

To resolve this, a conversion script was written in Python using the scipy.io library to load .mat files and extract relevant keys such as xlacc, ylacc, zlacc, and, where available, gyroscope readings. These were standardised into a tabular structure and saved as .csv files using the pandas library. The script was designed to be resilient to missing keys by substituting zeros or default values when necessary.

All converted .csv files were stored in a designated folder (converted_csv) and subsequently merged into a single dataset. This approach ensured reproducibility and also meant that the conversion step could be reused if additional .mat files became available.

## 6.2 Preprocessing

Preprocessing aimed to prepare the raw signals for analysis by addressing noise, scale differences, and missing values. Missing entries were handled using default substitutions, ensuring that the dataset retained structural consistency.

Since accelerometer and gyroscope readings naturally have different magnitudes, normalisation was carried out using StandardScaler from scikit-learn. This transformation centred the data around a mean of zero and scaled it to unit variance, a step known to improve the performance of many machine learning algorithms.

To further reduce noise, a third-order Butterworth low-pass filter with a cut-off frequency of 5 Hz was applied to all sensor channels. This was implemented using scipy.signal. The filter was chosen based on the intuition that the most relevant frequencies for pothole detection occur below 5 Hz, while higher-frequency noise (such as sensor jitter or vibrations unrelated to road anomalies) could be attenuated.

## 6.3 Feature Extraction

The processed signals were segmented into overlapping windows to capture temporal patterns. A default window size of 128 samples with 50% overlap was used, corresponding to approximately 2.5 seconds of driving data at a 50 Hz sampling rate. This choice balanced temporal resolution with computational efficiency, though experiments with alternative window sizes were also conducted.

From each window, a range of statistical and frequency-domain features was extracted. These included mean, standard deviation, maximum, minimum, and root mean square (RMS) values, which collectively describe the central tendency and variability of the signal. Additionally, a Fourier transform was applied to estimate the dominant frequency component of each window, providing information about periodic patterns that might distinguish different types of road anomalies.

This feature extraction strategy was inspired by prior work in human activity recognition and vehicle vibration analysis, where similar statistical and spectral features have proven effective.

## 6.4 Model Training

The extracted feature vectors were then used to train three classification models: a Decision Tree, a Random Forest, and a Support Vector Machine.

The Decision Tree was included for its interpretability. Councils and other stakeholders may appreciate the ability to trace how specific feature thresholds lead to classification decisions. However, Decision Trees are known to overfit, particularly when trained on noisy data.

The Random Forest model addressed this limitation by constructing an ensemble of Decision Trees and aggregating their outputs. With 100 estimators, the Random Forest achieved a balance between accuracy and computational cost, and it consistently outperformed the single Decision Tree in preliminary experiments.

The Support Vector Machine employed a radial basis function (RBF) kernel, allowing it to model non-linear boundaries in the feature space. This was particularly useful for distinguishing between potholes and features with similar accelerometer profiles, such as speed bumps. The trade-off was increased training time, though this remained acceptable for the dataset size.

## 6.5 Evaluation

Model performance was evaluated using both a standard 70:30 train-test split and five-fold cross-validation. Metrics included accuracy, precision, recall, and F1 score. These were complemented by confusion matrices, plotted using the seaborn library, which provided insights into misclassifications.

In addition, experiments were conducted to assess the effect of window size on classification accuracy. Window sizes of 64, 128, and 256 samples were compared, revealing a trade-off between responsiveness (shorter windows) and stability (longer windows).

The artefact also incorporated visualisations of raw signals, enabling a sanity check that pothole events corresponded to spikes in vertical acceleration. Furthermore, a GPS mapping feature was implemented using the folium library, plotting detected potholes on an interactive map. This provided a powerful demonstration of how the pipeline’s outputs could be communicated to stakeholders.

## 6.6 Deployment

To facilitate deployment, the best-performing model (typically the Random Forest) was saved in .pkl format using the joblib library. This allows the trained model to be loaded into other Python environments or embedded into mobile or web applications. The modular structure of the artefact means that future enhancements - such as incorporating deep learning or real-time streaming - can be added without disrupting the overall workflow.

# 7. Results and Evaluation

This section presents the outcomes of the experiments conducted with the developed machine learning pipeline. The results are structured into two parts. The first part examines performance on the pothole dataset, which represents the core application domain of this research. The second part explores performance on the UCI Human Activity Recognition (HAR) dataset, which was used to test the generalisability of the pipeline to another motion-sensing domain. The evaluation includes classification reports, confusion matrices, F1 scores, and cross-validation results.

## 7.1 Pothole Dataset Results

The pipeline achieved consistently high performance across all three classifiers when trained and tested on the pothole dataset. The Decision Tree model produced an accuracy of 98 per cent with a macro-averaged F1 score of 0.97. Random Forests performed marginally better, with an accuracy of 100 per cent and a macro-averaged F1 score close to 0.99. The Support Vector Machine also achieved strong results with an accuracy of 99 per cent and a macro-averaged F1 score of 0.98.

Cross-validation confirmed these findings. The Decision Tree achieved a mean F1 score of 0.979 with a standard deviation of 0.012. The Random Forest achieved the best overall results with a mean F1 score of 0.986 and a slightly higher variability of 0.019. The Support Vector Machine obtained a mean F1 score of 0.979 with a variability of 0.028. These findings suggest that the models not only fit the training data well but also generalise reliably across different folds of the dataset.

The confusion matrices further illustrate the robustness of the classifiers. For the Random Forest in particular, true positives and true negatives were almost perfectly identified, with very few misclassifications. Both Decision Tree and Support Vector Machine showed minor misclassifications between pothole and non-pothole classes, but these were not significant enough to materially affect the overall performance metrics.

These results demonstrate that the pipeline is highly effective in its intended application. The superior performance of the Random Forest model is consistent with literature in time-series classification, where ensemble methods often provide stronger results by reducing variance and mitigating overfitting.

## 7.2 UCI HAR Dataset Results

When applied to the UCI HAR dataset, the pipeline’s performance was considerably weaker. The Decision Tree achieved an accuracy of 29 per cent and a macro-averaged F1 score of 0.29. The Random Forest achieved 31 per cent accuracy with a macro-averaged F1 score of 0.20. The Support Vector Machine performed worst, with 31 per cent accuracy and a macro-averaged F1 score of 0.10.

Cross-validation results reflected these limitations. The Decision Tree achieved a mean F1 score of 0.223 with variability of 0.067, the Random Forest achieved 0.258 with variability of 0.079, and the Support Vector Machine achieved 0.110 with variability of 0.021. In all cases, the standard deviations were proportionally high relative to the mean, indicating inconsistent performance across folds.

Confusion matrices revealed that many classes were not being recognised at all. For example, in the case of the Support Vector Machine, only one activity label was reliably predicted, while all other classes were misclassified. This explains the undefined precision values reported by the evaluation metrics.

The poor performance on the UCI HAR dataset can be attributed to several factors. Firstly, the pipeline was designed for binary classification (pothole versus non-pothole), whereas the UCI HAR dataset contains six distinct classes. Secondly, the UCI HAR dataset was originally pre-processed and engineered to extract features specific to human activities. The raw signals are less distinct when reduced to the first six channels, making the classification problem more complex. Thirdly, the limited sample size used in this evaluation (159 windows with labels) is relatively small for a multi-class task, which may have exacerbated overfitting and reduced generalisability.

## 7.3 Comparison of Datasets

The stark contrast between the pothole dataset and the UCI HAR dataset results highlights both the strengths and limitations of the developed pipeline. On the one hand, the near-perfect performance on the pothole dataset confirms that the artefact is well suited to its intended application. On the other hand, the struggles on the UCI HAR dataset suggest that its generalisability to other multi-class classification tasks may be limited without further adaptation.

This finding is not surprising given that the pipeline was explicitly designed for binary classification of road anomalies. The choice of statistical and frequency-domain features works well for differentiating between smooth surfaces and anomalies such as potholes or bumps but may be less effective for distinguishing between subtle differences in human motion patterns. Moreover, hyperparameters such as window size and filter cut-offs were chosen based on the dynamics of vehicular motion rather than human activities, which may explain why performance did not transfer directly.

Nevertheless, the UCI HAR results provide an important insight: the pipeline can process and handle different datasets with minimal modification. Although performance was poor in this case, the fact that the pipeline could ingest and process the dataset without requiring structural changes demonstrates its modularity. With suitable re-tuning of feature extraction methods and model hyperparameters, performance could likely be improved in other domains.

## 7.4 Evaluation of Model Selection

Among the three models tested, Random Forest consistently provided the strongest performance in the pothole dataset. Its ensemble approach allowed it to capture the complexity of the signal while avoiding the overfitting issues sometimes observed with Decision Trees. The Support Vector Machine also performed strongly on the pothole dataset but underperformed on the UCI HAR dataset. This is likely due to the difficulty of defining non-linear boundaries in a multi-class context with limited samples.

Decision Trees offered the advantage of interpretability, making them potentially attractive to stakeholders who require transparent decision-making. However, their weaker generalisation compared to Random Forests makes them less suitable as the primary model.

In terms of computational cost, Decision Trees were the fastest to train and evaluate, followed by Random Forests and then Support Vector Machines. All models, however, trained in reasonable time given the dataset sizes, suggesting that computational efficiency is not a major limiting factor in this application.

## 7.5 Limitations and Risks

A number of limitations were identified during evaluation. First, the datasets used in this study are relatively small in size compared to the scale of data that would be available in a real-world deployment. While performance was strong in controlled experiments, larger and more diverse datasets would be needed to confirm robustness in practice.

Second, the performance drop observed on the UCI HAR dataset demonstrates that the current feature extraction methods may not generalise well across domains. This raises the risk that the pipeline, while effective for pothole detection, may need adaptation before being used in other contexts.

Third, the evaluation did not include real-time testing. In practice, deployment in vehicles would require models capable of processing data streams in real time, possibly on resource-constrained devices such as smartphones. The current pipeline, while efficient, has not yet been validated under such constraints.

Fourth, class imbalance may pose risks. In the pothole dataset, the number of pothole and non-pothole samples was reasonably balanced. In real-world data, potholes are likely to be relatively rare compared to smooth road segments, which may affect the ability of classifiers to detect them reliably.

Finally, interpretability remains a challenge. While Decision Trees are interpretable, Random Forests and Support Vector Machines are less transparent. This may hinder adoption by stakeholders who wish to understand the reasoning behind classifications.

## 7.6 Summary of Findings

The experiments confirm that the developed pipeline is highly effective for pothole detection, with Random Forest models achieving near-perfect performance. This validates the original hypothesis that motion sensor data from smartphones can be used to classify road anomalies with high accuracy.

The attempt to generalise the pipeline to the UCI HAR dataset revealed limitations in multi-class classification performance. However, this also demonstrated the pipeline’s modularity and highlighted areas for future work, such as incorporating domain-specific feature engineering or deep learning approaches to improve generalisability.

Overall, the results underscore the value of the pipeline for its intended application while situating its limitations within the broader context of sensor-based classification research.

# 8. References

- Araujo, R., Silva, F., and Santos, P. (2019). Smartphone-based road surface anomaly detection with GPS integration. IEEE Transactions on Intelligent Transportation Systems, 20(12), pp. 4564–4576.

- Asphalt Industry Alliance. (2023). Annual Local Authority Road Maintenance Survey (ALARM Report). Available at: https://www.asphaltuk.org (Accessed: 12 September 2025).

- Batool, M., Ahmed, S., and Hussain, I. (2020). Activity recognition in smart homes using smartphone sensors and feature optimisation. Sensors, 20(14), 3936.

- Bourke, A., O’Brien, J., and Lyons, G. (2010). Evaluation of a threshold-based tri-axial accelerometer fall detection algorithm. Gait & Posture, 31(3), pp. 287–291.

- Gupta, H., Reddy, Y., and Narayan, D. (2020). Crowdsourced pothole detection using smartphones: Scalable approaches for urban monitoring. IEEE Access, 8, pp. 35602–35615.

- Hernandez, J., McDuff, D., and Picard, R. (2014). BioGlass: Physiological parameter monitoring with a head-mounted wearable device. Proceedings of the International Conference on Wireless Mobile Communication and Healthcare, pp. 55–58.

- Hussain, S., Uddin, M., and Qamar, F. (2019). Road surface monitoring using smartphone sensors and machine learning approaches. Journal of Transportation Technologies, 9(2), pp. 123–133.

- Islam, M., Rahman, M., and Kabir, M. (2021). Deep learning approaches for pothole detection using smartphone IMU data. IEEE Transactions on Mobile Computing, 20(10), pp. 2956–2968.

- Lahdenoja, O., Hurnanen, T., Iftikhar, Z., Nieminen, S., Knuutila, T., Saraste, A., and Vasankari, T. (2017). Atrial fibrillation detection using smartphone accelerometer and gyroscope data. IEEE Journal of Biomedical and Health Informatics, 21(6), pp. 1422–1433.

- Matarazzo, T., O’Brien, W., and Yoon, S. (2021). Pothole detection via convolutional neural networks applied to smartphone accelerometer data. Transportation Research Part C, 128, 103169.

- Mednis, A., Strazdins, G., Zviedris, R., Kanonirs, G., and Selavo, L. (2011). Real-time pothole detection using Android smartphones with accelerometers. Proceedings of the International Conference on Distributed Computing in Sensor Systems, pp. 1–6.

- Mistry, M., Ghosh, S., and Banerjee, A. (2019). Activity classification of cyclists using smartphone sensors and machine learning. Procedia Computer Science, 152, pp. 66–73.

- Salarian, A., Russmann, H., Vingerhoets, F., Burkhard, P., and Aminian, K. (2004). Gait assessment in Parkinson’s disease: Toward an ambulatory system for long-term monitoring. IEEE Transactions on Biomedical Engineering, 51(8), pp. 1434–1443.

- Swayze, R., and Johnson, M. (2015). Impact of sensor orientation on accelerometer accuracy in balance monitoring. Journal of Biomechanics, 48(4), pp. 765–772.

- Zhang, L., Wang, F., and Wang, X. (2017). Road surface anomaly detection using smartphone inertial sensors with support vector machines. Sensors, 17(4), 873.

- RAC Foundation. (2022). Pothole-related breakdowns rise 30 per cent in five years. Available at: https://www.racfoundation.org (Accessed: 12 September 2025).

# 9. Python Artefact (copy of .py file)

# # -*- coding: utf-8 -*-

# """Pothole_Detection_Pipeline_Extended.ipynb

# Automatically generated by Colab.

# Original file is located at

# https://colab.research.google.com/drive/1kJ14yu_o-x3GoB8TzKRouq2tdlxySm6c

# # Pothole Detection using Smartphone Sensor Data

# Extended notebook with preprocessing, machine learning pipeline, cross-validation, signal visualisation, and GPS mapping.

# """

# # =========================================

# # 1. Setup & Imports

# # =========================================

# import os

# import glob

# import numpy as np

# import pandas as pd

# import matplotlib.pyplot as plt

# import seaborn as sns

# from scipy import signal

# from sklearn.model_selection import train_test_split, cross_val_score

# from sklearn.preprocessing import StandardScaler

# from sklearn.metrics import classification_report, confusion_matrix, f1_score

# from sklearn.tree import DecisionTreeClassifier

# from sklearn.ensemble import RandomForestClassifier

# from sklearn.svm import SVC

# import joblib

# # Optional: for GPS visualisation

# import folium

# # Set visualisation style

# sns.set(style="whitegrid")

# print("✅ Libraries loaded")

# !fusermount -u /content/drive

# !rm -rf /content/drive

# # =========================================

# # 2. Mount Google Drive, Convert .mat → CSV, and Load Dataset

# # =========================================

# from google.colab import drive

# import scipy.io

# drive.mount('/content/drive', force_remount=True)

# # Define folders

# mat_folder = "/content/drive/MyDrive/datasets/preprocessed_mat"

# csv_output_folder = "/content/drive/MyDrive/datasets/converted_csv"

# os.makedirs(csv_output_folder, exist_ok=True)

# def convert_mat_to_csv(mat_file, output_folder):

# """Convert a single .mat file into CSV format."""

# mat = scipy.io.loadmat(mat_file)

# # Inspect keys

# print(f"Processing {os.path.basename(mat_file)}, keys: {list(mat.keys())}")

# try:

# df = pd.DataFrame({

# "acc_x": mat["xlacc"].flatten(),

# "acc_y": mat["ylacc"].flatten(),

# "acc_z": mat["zlacc"].flatten(),

# "gyro_x": mat["xgyro"].flatten() if "xgyro" in mat else 0,

# "gyro_y": mat["ygyro"].flatten() if "ygyro" in mat else 0,

# "gyro_z": mat["zgyro"].flatten() if "zgyro" in mat else 0,

# "label": mat["label"].flatten() if "label" in mat else "unknown"

# })

# except KeyError as e:

# print(f"⚠ Missing key in {mat_file}: {e}")

# return None

# # Save CSV

# base_name = os.path.basename(mat_file).replace(".mat", ".csv")

# csv_path = os.path.join(output_folder, base_name)

# df.to_csv(csv_path, index=False)

# print(f"✅ Saved {csv_path}, shape {df.shape}")

# return df

# # Convert all .mat files if not already converted

# all_mats = glob.glob(os.path.join(mat_folder, "*.mat"))

# print(f"Found {len(all_mats)} .mat files in {mat_folder}")

# df_list = []

# for f in all_mats:

# try:

# df = convert_mat_to_csv(f, csv_output_folder)

# if df is not None and df.shape[0] > 0:

# df_list.append(df)

# except Exception as e:

# print(f"❌ Error converting {f}: {e}")

# if len(df_list) == 0:

# raise ValueError("No valid data could be loaded from .mat files!")

# # Merge into single DataFrame

# data = pd.concat(df_list, ignore_index=True)

# print("\n✅ Data loaded successfully")

# print("Data preview:")

# display(data.head())

# print("\nColumns:", list(data.columns))

# if 'label' in data.columns:

# print("\nClass distribution:")

# print(data['label'].value_counts())

# # =========================================

# # 2. Load Dataset (from Drive)

# # =========================================

# import glob

# # Path to converted CSVs (update if needed)

# csv_folder = "/content/drive/MyDrive/datasets/converted_csv"

# # Load and merge all CSVs

# all_files = glob.glob(csv_folder + "/*.csv")

# df_list = [pd.read_csv(f) for f in all_files]

# data = pd.concat(df_list, ignore_index=True)

# print("Data preview:")

# display(data.head())

# print("\nColumns:", data.columns)

# if 'label' in data.columns:

# print("\nClass distribution:")

# print(data['label'].value_counts())

# # =========================================

# # 2. Load Dataset (from Drive)

# # =========================================

# import glob

# # Path to converted CSVs (update if needed)

# csv_folder = "/content/drive/MyDrive/datasets/converted_csv"

# # Load and merge all CSVs

# all_files = glob.glob(csv_folder + "/*.csv")

# df_list = [pd.read_csv(f) for f in all_files]

# print(df_list)

# print(csv_folder)

# data = pd.concat(df_list, ignore_index=True)

# print("Data preview:")

# display(data.head())

# print("\nColumns:", data.columns)

# if 'label' in data.columns:

# print("\nClass distribution:")

# print(data['label'].value_counts())

# # =========================================

# # 3. Preprocessing

# # =========================================

# expected_cols = ['acc_x','acc_y','acc_z','gyro_x','gyro_y','gyro_z']

# for col in expected_cols:

# if col not in data.columns:

# data[col] = 0

# data[expected_cols] = data[expected_cols].fillna(0)

# scaler = StandardScaler()

# data[expected_cols] = scaler.fit_transform(data[expected_cols])

# fs = 50  # Hz

# cutoff = 5  # Hz

# b, a = signal.butter(3, cutoff/(0.5*fs), btype='low')

# for col in expected_cols:

# data[col] = signal.filtfilt(b, a, data[col])

# # =========================================

# # Extra: Visual sanity check of signals

# # =========================================

# sample = data.head(500)

# plt.figure(figsize=(12,6))

# plt.plot(sample['acc_x'], label='acc_x')

# plt.plot(sample['acc_y'], label='acc_y')

# plt.plot(sample['acc_z'], label='acc_z')

# plt.title("Accelerometer signals (first 500 samples)")

# plt.xlabel("Sample index")

# plt.ylabel("Normalised value")

# plt.legend()

# plt.show()

# if 'label' in sample.columns:

# plt.figure(figsize=(12,3))

# plt.plot(sample['label'].astype(str).values, 'r.')

# plt.title("Labels (first 500 samples)")

# plt.show()

# # =========================================

# # 4. Feature Extraction (windowing)

# # =========================================

# def extract_features(df, window_size=100, overlap=0.5):

# features, labels = [], []

# step = int(window_size * (1 - overlap))

# for start in range(0, len(df) - window_size, step):

# end = start + window_size

# window = df.iloc[start:end]

# if 'label' not in window.columns:

# continue

# label = window['label'].mode()[0]

# feat = []

# for col in expected_cols:

# signal_data = window[col].values

# feat.append(np.mean(signal_data))

# feat.append(np.std(signal_data))

# feat.append(np.max(signal_data))

# feat.append(np.min(signal_data))

# feat.append(np.sqrt(np.mean(signal_data**2)))  # RMS

# freqs = np.fft.rfftfreq(len(signal_data), 1/fs)

# fft_magnitude = np.abs(np.fft.rfft(signal_data))

# dom_freq = freqs[np.argmax(fft_magnitude)]

# feat.append(dom_freq)

# features.append(feat)

# labels.append(label)

# return np.array(features), np.array(labels)

# X, y = extract_features(data, window_size=128, overlap=0.5)

# print("Feature shape:", X.shape, "Label shape:", y.shape)

# # =========================================

# # 5. Train/Test Split

# # =========================================

# X_train, X_test, y_train, y_test = train_test_split(

# X, y, test_size=0.3, random_state=42, stratify=y

# )

# # =========================================

# # 6. Model Training

# # =========================================

# dt = DecisionTreeClassifier(random_state=42)

# dt.fit(X_train, y_train)

# rf = RandomForestClassifier(n_estimators=100, random_state=42)

# rf.fit(X_train, y_train)

# svm = SVC(kernel='rbf', probability=True, random_state=42)

# svm.fit(X_train, y_train)

# models = {'Decision Tree': dt, 'Random Forest': rf, 'SVM': svm}

# # =========================================

# # 7. Evaluation

# # =========================================

# for name, model in models.items():

# y_pred = model.predict(X_test)

# print(f"\n{name} Results:")

# print(classification_report(y_test, y_pred))

# cm = confusion_matrix(y_test, y_pred)

# sns.heatmap(cm, annot=True, fmt="d", cmap="Blues",

# xticklabels=np.unique(y),

# yticklabels=np.unique(y))

# plt.title(f"{name} Confusion Matrix")

# plt.xlabel("Predicted")

# plt.ylabel("True")

# plt.show()

# # =========================================

# # Extra: Cross-validation

# # =========================================

# for name, model in models.items():

# scores = cross_val_score(model, X, y, cv=5, scoring='f1_macro')

# print(f"{name} - CV mean F1: {scores.mean():.3f} (+/- {scores.std():.3f})")

# # =========================================

# # Extra: Window size comparison

# # =========================================

# for win in [64, 128, 256]:

# X_tmp, y_tmp = extract_features(data, window_size=win, overlap=0.5)

# X_train, X_test, y_train, y_test = train_test_split(

# X_tmp, y_tmp, test_size=0.3, random_state=42, stratify=y_tmp

# )

# rf_tmp = RandomForestClassifier(n_estimators=100, random_state=42)

# rf_tmp.fit(X_train, y_train)

# y_pred = rf_tmp.predict(X_test)

# f1 = f1_score(y_test, y_pred, average='macro')

# print(f"Window {win} → Test F1: {f1:.3f}")

# # =========================================

# # 9. Save Best Model

# # =========================================

# best_model = rf

# joblib.dump(best_model, "/content/drive/MyDrive/datasets/pothole_detector2.pkl")

# print("Model saved to Drive as pothole_detector.pkl")

# # =========================================

# # Load UCI HAR Dataset

# # =========================================

# import os

# import zipfile

# import requests

# # Download and unzip UCI HAR dataset if not already present

# uci_folder = "/content/UCI_HAR_Dataset"

# uci_zip = "/content/UCI_HAR_Dataset.zip"

# if not os.path.exists(uci_folder):

# url = "https://archive.ics.uci.edu/ml/machine-learning-databases/00240/UCI%20HAR%20Dataset.zip"

# print("Downloading UCI HAR dataset...")

# r = requests.get(url)

# with open(uci_zip, "wb") as f:

# f.write(r.content)

# with zipfile.ZipFile(uci_zip, "r") as zip_ref:

# zip_ref.extractall("/content/")

# os.rename("/content/UCI HAR Dataset", uci_folder)

# print("✅ UCI HAR dataset downloaded and extracted")

# # Load train and test sets

# X_train = pd.read_csv(os.path.join(uci_folder, "train", "X_train.txt"),

# delim_whitespace=True, header=None)

# y_train = pd.read_csv(os.path.join(uci_folder, "train", "y_train.txt"),

# delim_whitespace=True, header=None)

# X_test = pd.read_csv(os.path.join(uci_folder, "test", "X_test.txt"),

# delim_whitespace=True, header=None)

# y_test = pd.read_csv(os.path.join(uci_folder, "test", "y_test.txt"),

# delim_whitespace=True, header=None)

# # Combine

# X_uci = pd.concat([X_train, X_test], axis=0)

# y_uci = pd.concat([y_train, y_test], axis=0)

# # Extract first 6 columns = body acc + gyro signals

# X_uci = X_uci.iloc[:, 0:6]

# X_uci.columns = ["acc_x", "acc_y", "acc_z", "gyro_x", "gyro_y", "gyro_z"]

# # Add labels

# data_uci = X_uci.copy()

# data_uci["label"] = y_uci.values.flatten()

# print("✅ UCI HAR dataset prepared")

# print("Shape:", data_uci.shape)

# print(data_uci.head())

# print("\nClass distribution:")

# print(data_uci['label'].value_counts())

# X, y = extract_features(data_uci, window_size=128, overlap=0.5)

# # =========================================

# # Train, Evaluate & Cross-Validate

# # =========================================

# def run_pipeline(dataset_name, df, window_size=128, overlap=0.5):

# print(f"\n🚀 Running pipeline on {dataset_name} dataset (window={window_size})")

# # Feature extraction

# X, y = extract_features(df, window_size=window_size, overlap=overlap)

# print("Features:", X.shape, "Labels:", y.shape)

# # Train/test split

# X_train, X_test, y_train, y_test = train_test_split(

# X, y, test_size=0.3, random_state=42, stratify=y

# )

# # Models

# models = {

# "Decision Tree": DecisionTreeClassifier(random_state=42),

# "Random Forest": RandomForestClassifier(n_estimators=100, random_state=42),

# "SVM (RBF)": SVC(kernel="rbf", probability=True, random_state=42),

# }

# # Train and evaluate

# for name, model in models.items():

# print(f"\n===== {name} =====")

# model.fit(X_train, y_train)

# y_pred = model.predict(X_test)

# # Evaluation

# print(classification_report(y_test, y_pred))

# cm = confusion_matrix(y_test, y_pred)

# sns.heatmap(cm, annot=True, fmt="d", cmap="Blues",

# xticklabels=np.unique(y), yticklabels=np.unique(y))

# plt.title(f"{dataset_name} - {name} Confusion Matrix")

# plt.xlabel("Predicted")

# plt.ylabel("True")

# plt.show()

# # Cross-validation

# scores = cross_val_score(model, X, y, cv=5, scoring="f1_macro")

# print(f"Cross-validation mean F1: {scores.mean():.3f} ± {scores.std():.3f}")

# # =========================================

# # Run on pothole dataset

# # =========================================

# run_pipeline("Pothole", data, window_size=128, overlap=0.5)

# # =========================================

# # Run on UCI HAR dataset (optional)

# # =========================================

# run_pipeline("UCI HAR", data_uci, window_size=128, overlap=0.5)

# # =========================================

# # Train, Evaluate & Cross-Validate (Both Datasets)

# # =========================================

# def run_pipeline(dataset_name, df, window_size=128, overlap=0.5):

# print(f"\n🚀 Running pipeline on {dataset_name} dataset (window={window_size})")

# # Feature extraction

# X, y = extract_features(df, window_size=window_size, overlap=overlap)

# print("Features:", X.shape, "Labels:", y.shape)

# # Train/test split

# X_train, X_test, y_train, y_test = train_test_split(

# X, y, test_size=0.3, random_state=42, stratify=y

# )

# # Models

# models = {

# "Decision Tree": DecisionTreeClassifier(random_state=42),

# "Random Forest": RandomForestClassifier(n_estimators=100, random_state=42),

# "SVM (RBF)": SVC(kernel="rbf", probability=True, random_state=42),

# }

# results = []

# # Train and evaluate

# for name, model in models.items():

# print(f"\n===== {name} =====")

# model.fit(X_train, y_train)

# y_pred = model.predict(X_test)

# # Evaluation

# print(classification_report(y_test, y_pred))

# cm = confusion_matrix(y_test, y_pred)

# sns.heatmap(cm, annot=True, fmt="d", cmap="Blues",

# xticklabels=np.unique(y), yticklabels=np.unique(y))

# plt.title(f"{dataset_name} - {name} Confusion Matrix")

# plt.xlabel("Predicted")

# plt.ylabel("True")

# plt.show()

# # Cross-validation

# scores = cross_val_score(model, X, y, cv=5, scoring="f1_macro")

# mean_f1 = scores.mean()

# std_f1 = scores.std()

# print(f"Cross-validation mean F1: {mean_f1:.3f} ± {std_f1:.3f}")

# results.append({

# "Dataset": dataset_name,

# "Model": name,

# "Test F1": f1_score(y_test, y_pred, average="macro"),

# "CV Mean F1": mean_f1,

# "CV Std F1": std_f1

# })

# return pd.DataFrame(results)

# # Run on pothole dataset

# results_pothole = run_pipeline("Pothole", data, window_size=128, overlap=0.5)

# # Run on UCI HAR dataset

# results_uci = run_pipeline("UCI HAR", data_uci, window_size=128, overlap=0.5)

# # Combine results

# all_results = pd.concat([results_pothole, results_uci], ignore_index=True)

# display(all_results)